User’s Manual

V1.0

SAIM Scanner V3

by Marshall J. Colville

Paszek Research Group

# Table of Contents

1	Table of Contents	2

2	General Information	3

2.1	System Overview	3

2.2	Organization	4

2.3	System requirements	4

3	Hardware (SSv3 Controller)	5

3.1	Device layout	6

3.2	Connect accessories	7

4	Software (SSv3 Control Panel)	7

4.1	Install	8

4.1.1	Download	8

4.1.2	Pt grey (option)	8

4.1.3	VC runtimes	8

4.2	Controller	8

4.2.1	Overview	8

4.2.2	Menu	9

4.2.2.1	Settings	9

4.2.2.2	Logging	10

4.2.2.3	Demo Mode / About Device	11

4.2.2.4	About SSV3 device / SSv3 Control Panel	11

4.2.3	Controller Communications	12

4.2.3.1	Connecting	13

4.2.3.2	HID (USB human interface device)	13

4.2.3.3	Reset	14

4.2.4	Scanning Controls	15

4.2.4.1	Overview	16

4.2.4.2	Scanning beam Controls	16

4.2.4.3	Control Menu	16

4.2.4.4	Advanced Mode	16

4.2.4.4.1	Advanced settings	17

4.2.4.4.2	Calibration	17

4.2.5	Laser Controls	19

4.2.5.1	Overview	19

4.2.5.2	Laser Intensity	19

4.2.5.3	Shutter Control	19

4.2.5.4	SW Trigger	19

4.2.6	Laser Profiles	20

4.2.6.1	Overview	20

4.2.6.2	Laser Profiles Menu	20

4.2.7	Scan Angle Sequences	22

4.2.7.1	Overview	22

4.2.7.2	Sequence Menu	22

4.2.8	Experiment Design	24

4.2.8.1	Overview	24

4.2.8.2	Experiment Menu	24

4.3	Trouble shooting	26

5	Human Interface Device (HID)	26

# General Information

## System Overview

Scanning Angle Interference Microscopy Scanner V3 (SAIM Scanner V3, SSv3) is a device, which allows controlling a variety of peripheral devices. This device consists of two parts: SAIM Scanner V3 Controller and SAIM Scanner V3 Control Panel.

About SAIM Scanner V3 Controller

SSv3 controller is an embedded hardware synchronization platform for prototype super resolution optical microscopy. Currently, it is targeted for Scanning Angle Interference Microscopy (SAIM) using azimuthal beam scanning. The hardware has a wide array of capabilities and was developed with flexibility and expandability as a major project goal.

About SAIM Scanner V3 Control Panel

SSv3 control panel is designed to be an intuitive UI for operating the SSv3 controller. Provides flexible SAIM experiment design as well as real0time control for other imaging modalities.

## Organization

This user’s manual consists of two sections: Hardware and Software.

Hardware section provides a detailed description of Hardware components of this device and its accessories. This section includes 3 sub-sections: Device, Connect accessories and Adapter.

Software section explains how to install the program and use it with a detailed description of system functions. This section includes 8 sub-sections: Overview, Menu, Controller Communications, Scanning Controls, Laser Controls, Laser profiles, Scan Angle Sequences and Experiment Design.

## System requirements

Please check that your computer condition meets or exceeds the following system requirement before installing SAIM Scanner V3 Control Panel.

| Operating System | Windows 10, 8.1, 8, 7 |
| --- | --- |
| CPU |  |
| RAM | 256 MB? (2GB Recommended) |
| Available disk space | 200MB? |

# Hardware (SSv3 Controller)

SSv3 controller is an embedded hardware synchronization platform for prototype super resolution optical microscopy. Currently, it is targeted for Scanning Angle Interference Microscopy (SAIM) using azimuthal beam scanning. The hardware has a wide array of capabilities and was developed with flexibility and expandability as a major project goal.

Hardware section provides a detailed description of Hardware components of this device and its accessories.

## Device layout

| Name | Description |
| --- | --- |
| Power input | Power input (-12Vdc, 5Vdc, +12Vdc) |
| Aux 1 & 2 Output | Dual 10 bit high-speed bipolar analog outputs with 0-10 V adjustable references. |
| X & Y-axis output | Dual waveform outputs with high-speed independent per-axis amplitude and DC bias control |
| USB Type B input | USB HID interface for broad compatibility |
| RJ45 serial port | Reconfigurable serial communications port (UART, SPI) |
| AOTF Output | 8-channel 10 bit 0-10V analog outputs. |
| Status LED Header | 10V digital global blank |
| DIO Headers | 4 reconfigurable DIO pins. 3 interruptible inputs for hardware synchronization. |

## Connect accessories

Two accessories are available. Status LED Header and DIO Headers

# Software (SSv3 Control Panel)

SSv3 control panel is designed to be an intuitive UI for operating the SSv3 controller. Provides flexible SAIM experiment design as well as real0time control for other imaging modalities.

Software section explains how to install the program and use it with a detailed description of system functions

## Install

Install section explains how to install the program: Download, Unzip, save path, setting for pt grey and VC runtimes.

### Download

SSv3 Control Panel is available from https://github.com/mjc449/SAIMscannerV3

1. Direct download: Click ‘Download ZIP’

2. Git Bash: Clone with HTTPS: https://github.com/mjc449/SAIMscannerV3.git

### Pt grey (option)

Flycapture sdk version

### VC runtimes

Download the latest version Microsoft website.

## Controller

Controller section provides how to use the SAIM Scanner V3 Control Panel. This section includes a detailed description for Overview, Menu, Controller Communications, Scanning Controls, Laser Controls, Laser Profiles, Scan Angle Sequences, and Experiment Design.

### Overview

SAIM Scanner V3 Control Panel is a Graphic User Interface software, which consists of 7 sub-sections. Menu section can save/load the settings of Control Panel and run the experiment logger. Controller Communications section can connect/disconnect with a SAIM Scanner V3 and check the connection status of the product.  Other 6 sections can control laser profile, scanning setting, scan angle setting, and experiment design.

### Menu

Menu section provides information about software settings, experiment logger. This section consists of Settings, Logging, Demo Mode, and About SSV3 device / SSv3 Control Panel.

#### Settings

Settings saves SAIM Scanner V3 control panel configuration. The following values are saved.

- Calibration constant, X and Y Center, TIR radius, Phase, Frequency, Y Scale

To save settings,

1. Click ‘File’ and ‘Save settings’

2. Set the directory and save the ‘settings.cfg’

To load settings,

1. Click ‘File’ and ‘Load settings sheet’

2. Select the ‘setting.cfg’ file what you want to load.

(can be used to restore to reset) -> reset funtion

#### Logging

To Identify any errors that occur during the experiment or Identify any experimental records, recommend using Experiment Logger

To start logger,

1. Click ‘Logging’ and ‘Experiment log’

2. Click ‘Start/Stop error Logging’ to start/stop logger.

- ‘Add / Clear Comment’: Add/Clear any comments on the experiment.

- ‘Save log as’: Save log file as.

- ‘Autosave’: Click the check-box and select the directory to save log file.

- ‘Record elements’: Save elements log.

- ‘Record settings’: Save settings log.

#### Demo Mode / About Device

1. Click ‘Device’ and ‘Demo mode’ to access Demo mode.

2. Demo mode can control SSv3 Control Panel without connection to SAIM Scanner V3.

#### About SSV3 device / SSv3 Control Panel

### Controller Communications

Controller Communications section explains how to connect/disconnect to a product

#### Connecting

When SAIM Scanner product is connected to your computer, display panel shows product information: Serial Number, Product, Serial Number.

To connect to SAIM Scanner product, follow these steps:

1. Click the product what you want to connect to. (If your product is not on the list, please check connection between your computer and the product and click ‘Refresh’)

2. Click ‘Connect’. If the status shows ‘OK’, your computer and the product are well connected.

(If it shows ‘Error’, please check the connection between computer and product.)

3. Click “Disconnect’ when your experiment is done.

#### HID (USB human interface device)

1. Click ‘HID’ to access USB human interface device.

2. Click ‘Red pill’ to open direct communications with the controller.

To control the controller with HID, please go to Section 5. HID

#### Reset

Click ‘Reset’ to reset the device. The stored data will be erased, and the device is restored to the default setting.

### Scanning Controls

Scanning Controls section explains how to control scanning beam. This section consists of Scanning beam controls, Control menu, Advanced mode, and Calibration.

#### Overview

#### Scanning beam Controls

Control scan radius and X- & Y-park with DAC or angle value. These values are be input via mouse or keyboard.

#### Control Menu

In this menu, 4 control buttons can be used to control scanning beam.

Scan: Turn on/off ‘the Scan Mode’.

Center Park: Set X,Y value to the center.

Location Park: Turn on/off ‘Location Park Mode’

TIR: Total Internal Reflection Angle

#### Advanced Mode

Click ‘Advanced’ button, Advanced mode is activated.

##### Advanced settings

1. Set TIR radius / X,Y Center / Phase / Y-scale values.

2. Change the Frequency of Galvano mirror.

##### Calibration

Scanning Angle Interference Microscopy, a fluorescence-based nanoscale interferometric approach, is very sensitive to alignment. Calibration should be done for every experiment to obtain precise values.

- Click ‘Calibration’ button, 1D calibration window will be opened.

- Calibration window consists of

1. Calibration Display: Any pictures or camera can be displayed in this area. All images can be expanded by mouse-dragging

2. Fitting Control: To fit well, you can adjust all parameters: Kernel, Threshold, Accumulator, Length, Gap, Dilate, and Erode. Click ‘Detect Line’ to fit the lines as shown below. The angle between two straight lines is automatically shown in DAC & Angle Sheet.

Tips for fitting

- Adjust Kernel & Threshold values so that the two straight lines are visible well.

- Magnify the image by mouse-dragging.

- Adjust Accumulator, Length, Gap values until the two straight lines fit well.

- If fitting does not work, please use the ‘Dilate’ and ‘Erode’.

- Recommended to measure angle value with the DAC from 1000 to 13000.

### Laser Controls

Laser Controls section explains how to control laser intensity and shutter. This section consists of Laser Intensity, Shutter Control, and Software Trigger

#### Overview

#### Laser Intensity

7 lasers (405 nm, 445 nm, 488 nm, 515 nm, 535 nm, 560 nm, and 642 nm.) can be controlled. This intensity value is not the absolute value but a relative intensity value.  For example, ’50’ means 50% of original laser intensity.

#### Shutter Control

1. On/Off: Control System On/Off.

2. Fire On/Off: The shutter opens only when the CCD or camera is operating.

3. Open/Closed: Shutter opens/closed when this button is ‘Open’/’Closed’ .

4. Reset:

5. Zero All: Set all laser intensity to 0 value.

#### SW Trigger

Click ‘SW Trigger” after entering the trigger time.

### Laser Profiles

Laser Profiles section provides how to set profiles for lasers. This section consists of Profiles Display, and Profiles Menu.

#### Overview

#### Laser Profiles Menu

1. Click ‘New’ to make new laser profile.

2. Select the laser wavelength and intensity.

3. Click ‘OK’ (Click ‘Clear’ if you want to clear the profiles)

4. The laser profile you chose is shown in the Profile Display below.

Or, you can make new laser profile through Laser Control.

5.  Select the laser type and intensity in the Laser Control.

6. Click ‘Add’ and the laser profile you chose is shown in the Profile Display.

7. Click ‘Update’ when you want to change the laser profile.

### Scan Angle Sequences

Scan Angle Sequences section provides how to set scan angle sequences. This section consists of Sequence Display, and Sequence Menu.

#### Overview

#### Sequence Menu

1. Click ‘New’ to add new scan angle sequence.

2. When a new window opens, choose unit (Degree or DAC) on the right.

3. Input start angle, stop angle and number.

- Start angle: Laser angle at the beginning.

- Stop angle: Laser angle at the end.

- Number: The step number between start angle and stop angle.

4. The scan angle sequence (sequence #, type, first angle, last angle, and step count) is shown in the Sequence Display

5. Click ‘Modify’ when you want to change the sequence.

6. Use ‘Custom’ when you want to use custom sequence not linear scan angle sequences.

### Experiment Design

Experiment Design section explains how to design the experiment with Laser Profiles and Scan Angle Sequence. This section consists of Experiment Display, and Experiment Menu.

#### Overview

#### Experiment Menu

1. Click ‘Add’ to add new experiment.

2. When a new window opens, select step number, angle sequence, and laser profile.

- Step Number: Experiment Number

- Angle Sequence: Scan Angle Sequence what you want to use

- Laser Profile: Laser Profile what you want to use

3. Click ‘Add’ once you have made your selection.

4. Your experiment (sequence #, Profile #) is shown in the Experiment Display.

5. Click ‘Start’ / ‘Stop’ to start/stop the all experiments. Experiment goes from top to bottom.

6. Modify your experiment (‘Up’ / ‘Down’ / ‘Delete’ / ‘Clear All’)

- ‘Up’ / ‘Down’: change the order of experiments.

- ‘Delete’: delete the experiment

- ‘Clear All’: delete all experiments.

7. Click ‘Loop’ to use one experiment repeatedly. Star (*) is shown in the experiment.

## Trouble shooting

If the tips below do not solve your problem, please contact Paszek Research Group

(Marshall J. Coville)

# Human Interface Device (HID)

//Command ID defines

//0x0X are control commands

#define CMD_BLINK_PWR      0x00

#define CMD_VISOR          0x01

#define CMD_BLINK_PWR_VAR  0x02

//0x1X are scan commands

#define CMD_SET_RADIUS     0x10

#define CMD_SET_CTR        0x11

#define CMD_SET_TIRF       0x12

#define CMD_CIRCLE_SCAN    0x13

#define CMD_TIRF_SCAN      0x14

#define CMD_LOC_PARK       0x15

#define CMD_DISC_SCAN      0x16

#define CMD_DISC_SCAN_OFF  0x1E

#define CMD_CENTER_PARK    0x1F

//0x2X are DDS commands

#define CMD_SET_FREQ       0x20

#define CMD_DEFAULT_FREQ   0x21

#define CMD_SET_PHASE      0x22

#define CMD_DEFAULT_PHASE  0x23

#define CMD_MCLK_TOGGLE    0x24

#define CMD_WAVE_RS        0x25

#define CMD_WAVE_CLR_RS    0x26

//0x3X are AUX_DAC commands

#define CMD_CONST_AUX      0x30

#define CMD_MID_AUX        0x31

#define CMD_ZERO_AUX       0x32

//0x4X are AOTF commands

#define CMD_GLOBAL_HIGH    0x40

#define CMD_GLOBAL_LOW     0x41

#define CMD_CHANGE_CH      0x42

#define CMD_LOAD_PROFILE   0x43

#define CMD_OPEN_SHUTTER   0x44

#define CMD_CLOSE_SHUTTER  0x45

#define CMD_TOGGLE_OPEN    0x46

#define CMD_ADD_PROFILE    0x4C

#define CMD_DELETE_PROFILE 0x4D

#define CMD_DEL_ALL_AOTF   0x4E

#define CMD_AOTF_RESET     0x4F

//0x5X are interrupt commands

#define CMD_FIRE_ON        0x50

#define CMD_FIRE_OFF       0x51

//0x6X are triggering commands

//0x7X are SC <-> SC communications

//0x8X are experiment commands

#define CMD_GET_SEQ_USB    0x80

#define CMD_DEL_SEQ        0x81

#define CMD_ADD_SEQ_LIN    0x82

#define CMD_ADD_EXP        0x83

#define CMD_ADD_NODE_START 0x84

#define CMD_ADD_NODE_END   0x85

#define CMD_ADD_LOOP       0x86

#define CMD_START_EXP      0x87

#define CMD_PAUSE_EXP      0x88

#define CMD_RESUME_EXP     0x89

#define CMD_RESTART_EXP    0x8A

#define CMD_DEL_EXP        0x8B

#define CMD_DEL_NODE_START 0x8C

#define CMD_DEL_NODE_END   0x8D

#define CMD_COUNT_STEPS    0x8E

#define CMD_STOP_EXP       0x8F

//0x9X are SimpleSAIM commands

#define CMD_LOAD_SIMPLE_HALF  0x90

#define CMD_LOAD_SIMPLE_FULL  0x91

#define CMD_DIRECTION         0x92

#define CMD_START_SIMPLE      0x93

#define CMD_START_DITHERED    0x94

#define CMD_STOP_SIMPLE       0x95

#define CMD_STEP_COUNT        0x96

//0xAX are Mirror Detector commands

#define CMD_SET_MD_RADIUS  0xA0

#define CMD_MD_ON          0xA1

#define CMD_MD_OFF         0xA2

//0xFX are special function commands

#define CMD_TS_PERIOD      0xF0

#define CMD_CHECK_MEM      0xFD

#define CMD_SEND_STAT      0xFE

#define CMD_RESET_CPU      0xFF